# Lab 3: Differential Pulse-Code Modulation (DPCM)

## Objective

To understand and implement Differential Pulse-Code Modulation (DPCM), study lossless and lossy predictive coding, compare open-loop versus closed-loop prediction, analyze quantization effects, and compute error metrics such as MSE and PSNR.

## Part 1: Lossless DPCM on Image Data

### Step I — Linear Representation of Image Data

Convert a 2D grayscale image matrix into a 1D sequence by flattening rows in raster-scan order. This representation enables sequential predictive processing.

### Step II — Inverse Linear Representation

Reconstruct the original 2D image by reshaping the 1D linear sequence back into its height × width structure.

### Step III — Lossless DPCM Encoding

Lossless DPCM reduces redundancy using a simple predictor. The prediction is:
   x̂[n] = x[n−1]
The difference signal is:
   d[n] = x[n] − x̂[n]
The first sample must be transmitted explicitly since it has no predictor.

### Step IV — Lossless DPCM Decoding

Reconstruction is performed by summing the previous reconstructed value with the difference:
   x̃[n] = x̃[n−1] + d[n]
This results in perfect reconstruction.

## Part 2: Lossy DPCM on Image Data (Quantization)

Lossy DPCM introduces a quantizer after the difference computation to reduce bitrate, at the cost of reconstruction accuracy.

### Quantizer Step Size

For 8-bit images, the amplitude range of the difference signal is approximately −128 to +128. The uniform quantizer step size Δ is:
   Δ = 256 / (2^B)
where B is the number of bits used to represent each quantized difference.

### Closed-Loop DPCM Encoding

Closed-loop encoding uses the reconstructed (quantized) previous value as the predictor:
   x̂[n] = x̃[n−1]
This ensures that both encoder and decoder use the same prediction reference, which prevents quantization error from accumulating across samples.

### Open-Loop DPCM Encoding

Open-loop encoding uses the original previous sample for prediction:
   x̂[n] = x[n−1]
However, the decoder reconstructs using quantized differences, creating a mismatch:
   x̃[n] = x̃[n−1] + d̂[n]
This mismatch causes quantization error to accumulate from sample to sample, resulting in noticeable degradation.

### Decoding (Both Modes)

Regardless of open- or closed-loop encoding, the decoder reconstructs the signal using:
   x̃[n] = x̃[n−1] + d̂[n]
The difference between schemes arises solely from the predictor used during encoding.

## Error Metrics: MSE and PSNR

Mean Squared Error (MSE) measures the average squared difference between original and reconstructed signals:
   MSE = (1/N) Σ (x[n] − x̃[n])²

Peak Signal-to-Noise Ratio (PSNR) measures reconstruction quality relative to signal amplitude:
   PSNR = 10 log10 (MAX² / MSE)
For 8-bit images, MAX = 255.

## Part 3: Application to 1D Audio Signal

This section applies DPCM concepts to real audio data to visualize error reduction and quantization noise.

### Step I — Load and Normalize Audio

Load a WAV audio file, extract the samples, and normalize them to a consistent range (e.g., −1 to +1).

### Step II — Lossless DPCM on Audio

Apply lossless DPCM encoding. The difference signal will show reduced dynamic range compared to the original waveform.

### Step III — Lossy DPCM with Different Bitrates

Apply lossy closed-loop DPCM with two bit settings:
• High Quality: B = 4 bits per difference (16 levels)
• High Compression: B = 2 bits per difference (4 levels)
Lower bit depth introduces more quantization noise, especially in slowly varying regions.